LDH 5-WAY MANIFOLD

A Task Force Tips model # AL92P2P3 5-way manifold shall be provided. The unit shall be configured with a 4" Storz swivel inlet, and 4" Storz swivel outlet with a slow open/close valve with only 2 PSI friction loss at 1000 GPM. The Storz couplings shall be easily configurable to swivel from rigid. The unit shall have four (4) 2-1/2" male NH outlets with quarter-turn aluminum ball valves with full 2-1/2" waterway, and color-coded folding Nylon handles with auto-lock feature. A carrying handle shall be standard. The unit shall be equipped with an automatic pressure relief valve that is adjustable up to 300 psi. A protected pressure gauge shall be included. For corrosion resistance the unit shall be hardcoat anodized aluminum alloy and shall have a silver powder coat interior and exterior finish. Meets NFPA 1965 900 PSI hydrostatic testing. The unit shall have a unique serial number and be covered by a five-year warranty.