IN-LINE VALVE

Task Force Tips model #AN5R2P01 manually operated lightweight aluminum ball valve shall be provided. The valve shall be controlled with an NFPA compliant slow-close hand wheel gear operator which can be configured for left or right hand operation. A position indicator shall be provided to allow for quick visualization of the status of the valve in the open, closed or partial positions. The unit shall have a large carrying handle. For maximum corrosion protection the aluminum casting shall be hardcoat anodized, with a powder coat internal and external finish and the valve ball shall be stainless steel. The unit shall be equipped with blank plate where an adjustable pressure relief valve may be added.

The connections shall be: 4-1/2" NH female long handle swivel inlet and 4” Storz swivel outlet and include polymer bearing strips for prevention of galvanic corrosion. The unit shall have a unique serial number and be covered by a five-year warranty.