INDUSTRIAL VALVE UNDER MONITOR

Task Force Tips model #ZAAD3 manually operated industrial valve under monitor shall be provided. The unit shall be equipped with an external, automatic drain valve to drain the main valve body and monitor. The valve shall be controlled with an NFPA compliant slow-close hand crank gear operator which can be configured for left or right hand operation. Dual, reflective valve position indicator shall be provided to allow for quick visualization of the status of the valve in the open, closed or partial positions. For maximum corrosion protection the aluminum casting shall be hardcoat anodized, with an American red powder coat internal and external finish. The valve ball shall be hardcoat anodized 6061-T6 aluminum alloy and have an automatic drain for draining waterway when valve is closed and unpressurized.

The connections shall be: 4” ANSI 150 /DN80 PN16 flange inlet and 4” ANSI 150 /DN80 PN16 flange outlet. The unit shall have a unique serial number and be covered by a five-year warranty.