
IN-LINE VALVE
Task Force Tips model #AN3P7R01 manually operated lightweight aluminum ball valve shall be provided. The valve shall be controlled with an NFPA compliant slow-close hand wheel gear operator which can be configured for left or right hand operation. A position indicator shall be provided to allow for quick visualization of the status of the valve in the open, closed or partial positions. The unit shall have a large carrying handle. For maximum corrosion protection the aluminum casting shall be hardcoat anodized, with a powder coat internal and external finish and the valve ball shall be stainless steel. The unit shall be equipped with blank plate where an adjustable pressure relief valve may be added. A 3/4" female NPT threaded and plugged port shall allow the addition of an optional bleeder valve to exhaust excess air or water from the valve and hoseline.
The connections shall be: 4" NH female rocker lug swivel inlet and 4-1/2” NH male rigid outlet and include polymer bearing strips for prevention of galvanic corrosion. The unit shall have a unique serial number and be covered by a five-year warranty.

