
DUAL AGENT NOZZLE
Task Force Tips model # B-DC2060BC dual agent (two inlets) nozzle shall be provided. The nozzles shall be designed to deliver dry chemical powder at a rate of five pounds per second and water or foam solution at a rate of 20 and 60 GPM at 100 PSI. The front portion of the dry chemical nozzle shall be designed with a straight through path, include straightening fins and withstand the abrasive nature of dry chemical powder. The water/foam solution Bubble Cup nozzle shall be configured for flow settings of 20 and 60 GPM at 100 PSI and be capable of producing either a fog or straight stream with the ability to instantly aspirate foam by sliding a sleeve forward.
For corrosion resistance and durability the nozzle shall be constructed from hardcoat anodized aluminum alloy, the dry chemical nozzle shall have six (6) position detent flow control stainless steel slide valve. The foam/water nozzle shall have a stainless steel ball valve. A single pistol grip handle shall be positioned below the nozzles.
The nozzle shall have a 1" female NH swivel coupling for the dry chemical powder and a 1" female NH swivel rocker lug coupling for the water/foam inlet. The unit shall have a unique serial number and be covered by a written five-year warranty.

