
COMPRESSED AIR FOAM NOZZLE
Task Force Tips model # H-VITGC compressed air nozzle with shutoff shall be provided. The shutoff shall be designed with a six (6) position detent (click stop) flow control handle. For corrosion resistance the valve body shall be constructed from hardcoat anodized aluminum extruded alloy. An integral white pistol grip handle shall be positioned directly below the valve handle. The valve bail handle shall have white handle covers. The valve body shall be designed to accept user interchangeable smooth bore inserts which shall be included. The inserts shall be made from polymer material and have the size of the orifice laser engraved on the insert. The nozzle shall be suitable for foam solution and compressed air foam application.
The following sizes shall be supplied; 7/8", 15/16", 1", 1-1/8" inserts or 1-3/8" waterway without insert.

The nozzle shall be designed to be used with 1-1/2" through 2" attack lines. The couplings shall be configured as a 1-1/2" female NH swivel inlet with the ability to continuously swivel when tightened and a 1-1/2" male NH outlet and be designed for use with Task Force Tips CAFS-Force combination fog/straight stream nozzle tip, smooth bore tip, stack tip, or used for break-and-extend operations.

