
DUAL PRESSURE AUTOMATIC NOZZLE-PLAYPIPE
Task Force Tips Dual-Force series, model # HD-2BLITZ dual pressure automatic nozzle/playpipe with shutoff shall be provided. A knob located at the front of the nozzle shall allow for pressure rating adjustments of 100 or 55 PSI. The nozzle design shall allow for straight stream through dense wide fog patterns and be able to be flushed without shutting down. For corrosion resistance and durability the nozzle shall be constructed from hardcoat anodized aluminum alloy, have a six (6) position detent flow control stainless steel slide valve, a rubber bumper with fog teeth, stainless steel inlet debris screen and reflective labeling. Wrap around, rubber coated handles designed to reduce fatigue shall be provided.

The nozzle shall have a 2-1/2" female NH swivel inlet that allows the nozzle to swivel when tightened, a flow range of 95-300 GPM at 100 PSI and be designed to accept the Task Force Tips FJ-H low expansion or FJ-HMX multi expansion FoamJet foam attachments.

