
COMPRESSED AIR FOAM DUAL LOW PRESSURE AUTOMATIC NOZZLE
Task Force Tips CAFS-Force, model # HMDC-TO dual pressure automatic compressed air foam nozzle tip shall be provided. A knob located at the front of the nozzle shall allow for pressure rating adjustments of 75 or 45 PSI. The nozzle design shall allow for straight stream through dense wide fog patterns and be able to be flushed without shutting down. The nozzle shall be marked for "WATER" in the 75 psi setting and "CAFS" in the low pressure setting. The nozzle shall be suitable for foam solution and compressed air foam application.
For corrosion resistance and durability the nozzle shall be constructed from hardcoat anodized aluminum alloy, shall have a protective rubber bumper with fog teeth, laser engraved serial number, reflective labeling and five year warranty. The nozzle shall comply with the current NFPA 1964 standard.

The nozzle shall have a 1-1/2" female NH inlet with rocker lug coupling, a flow range of 70-200 GPM at 75 PSI and be designed to accept the Task Force Tips FJ-LX-HM low expansion or FJ-MX-HM multi expansion FoamJet foam attachments.

