
FIXED GALLONAGE NOZZLE
Task Force Tips Metro 2, model # ME2-2TO fixed gallonage nozzle tip shall be provided. The nozzle design shall allow for straight stream through wide fog patterns with user selectable flows and pressures and be able to be flushed without shutting down. For corrosion resistance and durability the nozzle shall be constructed from hardcoat anodized aluminum alloy, a rubber bumper with "power fog" teeth, stainless steel inlet debris screen and reflective labeling. The nozzle shall conform to the current NFPA 1964 standard.
The nozzle shall have user changeable flow discs with the following flow and pressure capabilities:
250 GPM @ 50 PSI (configuration when shipped)
125 GPM @ 75 PSI
185 GPM @ 75 PSI
200 GPM @ 75 PSI
250 GPM @ 75 PSI
175 GPM @ 100 PSI
250 GPM @ 100 PSI
325 GPM @ 100 PSI
The nozzle shall have a 2-1/2" female NH rigid rocker lug inlet and be designed to accept the Task Force Tips FJ-H low expansion or FJ-HMX medium expansion foam attachments.

