
HIGH FLOW MASTER STREAM NOZZLE
Task Force Tips Master Stream 4000 series model # MT-R-6SX selectable pressure master stream nozzle shall be provided. Designed for use on fire boats, large truck mounted monitors and industrial applications, the nozzle shall have a flow range of 600 to 4000 GPM with an adjustable pressure operating range anywhere from 80 to 120 PSI. The pressure adjustment control knob shall be located at the front of the nozzle. The nozzle shall maintain constant pressure throughout the flow range and deliver a variable stream pattern from straight to wide fog.
For corrosion resistance, the nozzle shall be constructed from hardcoat anodized aluminum alloy and include a cast aluminum bumper with integral fog teeth to produce a finger free fog pattern. A halo ring shall be provided to assist with stream pattern adjustment.
The nozzle shall be configured with a 6" Storz rigid inlet.

