
WATER THIEF
A Task Force Tips model # WT5HNJ-NJ-NF water thief shall be provided. The lightweight appliance shall be configured with a 2-1/2" female NH swivel long handle coupling inlet, a 2-1/2" male rigid outlet and two (2) 1-1/2" male NH side port outlets. The outlets shall have quarter-turn aluminum ball valves and color-coded folding Nylon handles. The main 2-1/2" valve shall have an auto-lock feature. The main waterway shall have a full 2-1/2" waterway and side ports shall have 1-1/2" waterways. A 1/4" NPT female plugged pressure gauge port and carrying strap shall be standard. For corrosion resistance the water thief shall be hardcoat anodized and shall have a durable silver powder coat interior and exterior finish. The unit shall be covered by a five-year warranty.

