
SAFE-TAK PORTABLE MONITOR PACKAGE
Task Force Tips Crossfire model # XFC-52 portable lightweight monitor package consisting of monitor top, Master Stream 1000 or 1250S series nozzle, stacked tips, stream straightener, Safe-Tak portable ground base, and base storage bracket shall be supplied.
Task Force Tips Crossfire, model portable monitor top shall be provided. This top only portion with quick release swivel joint shall be designed for use on truck mounted risers and TFT Safe-Tak 1250 series portable bases. The monitor shall include safety devices that include a locking button which locks the quick release lever when monitor is pressurized, and a 1/4 turn rotational lever lock that secures the horizontal rotation and provides a visual indication that the monitor rotation is locked. For corrosion resistance the monitor shall be constructed from hardcoat anodized aluminum with a red powder coat interior and exterior finish.
The monitor shall have a 3-1/4" waterway for delivery of up to 1250 GPM with low friction loss. Vertical elevation shall be controlled through use of a handwheel controlled stainless steel worm gear which allows full travel to the safety stop point of 35 degrees above horizontal with seven rotations of the wheel. When positioned on a truck mounted riser the monitor shall be able to be used below the 35 degree stop point through release of the spring loaded safety pin. An automatic drain to remove remaining water and avoid freezing shall be included. Integral stainless steel stream straightener and pressure gauge shall be included. 
Task Force Tips Safe-Tak 1250, portable monitor base shall be provided. The monitor base shall include a Safe-Tak, spring loaded butterfly valve designed to rapidly reduce the water flow by 90 percent in the event that contact with the ground is lost. The device shall include an integral carrying handle, four folding stainless steel legs with replaceable tungsten carbide spikes and an anchoring strap attached to a protective cap designed to be stored inside the waterway. The butterfly valve shall have a reset handle located near the inlet to allow the water flow to be reestablished once the base is properly stabilized. 
The base shall be constructed from hardcoat anodized aluminum and have a red powder coat interior and exterior finish. The base shall have either a single inlet or a dual hose inlet with clapper valve. A storage bracket for the portable base shall be included.
Task Force Tips Master Stream series nozzle shall be provided. The nozzle shall be designed for use on monitors, ladder pipes, deluge guns and aerial platforms. For corrosion resistance the nozzle shall be constructed for lightweight hardcoat anodized aluminum. The nozzle shall have a UV resistant rubber bumper with integral teeth designed to produce a finger free fog pattern. A halo ring shall be included to assist with stream shape control. The nozzle shall be suitable for foam solution application and designed to accept the Task Force Tips FJ-LX-M low expansion air aspirating attachment. The nozzle shall be configured with a female swivel rocker lug coupling.
Task Force Tips smooth bore stacked tip set shall be provided. For corrosion resistance the tip set shall be constructed from hardcoat anodized aluminum alloy. The set shall consist of four (4) tips with the base tip having a 2-1/2" female NH swivel inlet and 2" outlet. The other tip sizes shall be 1-3/4", 1-1/2" and 1-3/8". Each tip shall be laser engraved with a flow/pressure chart, orifice size, and thread size.
Task Force Tips stream straightener shall be supplied. The straightener shall be constructed from extruded aluminum with internal vanes designed to reduce turbulence and increase the reach of smooth bore water streams. The device shall have 2-1/2" female NH rigid inlet and 2-1/2" male NH rigid outlet. The primary components shall each have a unique serial number and all components shall be covered by a five-year warranty.
Specify portable base model number:

Specify stream straightener model number:
Specify nozzle model number:


