
PORTABLE MONITOR BASE
Task Force Tips Stow-A-Way 800, model # XFL-2NJ portable monitor base shall be provided. The device shall include an integral carrying handle, four folding stainless steel legs with replaceable tungsten carbide spikes and an anchoring strap (attached to a protective cap) designed to be stored inside the waterway.
The base shall be constructed from hardcoat anodized aluminum and have a red powder coat interior and exterior finish. The inlet shall be configured with two (2) 2-1/2" female NH swivel rocker lug couplings with two-way clapper valve. The unit shall be covered by a five-year warranty.

