
REMOTE CONTROL TELESCOPING MONITOR PIPE
Task Force Tips model # XGA38VL-RL24 3" electrically telescoping waterway compatible with 24 volt DC systems shall be installed. The waterway shall be capable of being lowered to deck level (or into a monitor well) for storage and transportation and shall be capable of being raised to an extended height of 18" using Auxiliary 1 push button and retract after the monitor has been parked by pressing the Park button on Task Force Tips RC monitor control stations or  from optional panel mounted push buttons. The unit shall include a 24 volt motor capable of moving the waterway to either the raised or lowered position while maintaining the ability to horizontally rotate the monitor device 360 degrees. The motor shall be weatherproof in design and have an accessible manual override control for use in the event power failure occurs. An interface box with cables and waterproof plugs shall be provided for connection of power, monitor, Extend-A-Gun RC and optional push button controls.
A sensor shall be located on the waterway that signals a 24 volt indicator light installed in the cab to illuminate to indicate that the monitor is raised.
The aluminum riser shall have a 3" waterway; hardcoat anodized finish and be furnished with a 3" Victaulic inlet and a TFT Code RLM male connection for a TFT remote control monitor with TFT Code RLF female inlet. The unit shall be covered by a five-year warranty.

