
OSCILLATING PORTABLE ATTACK MONITOR
Task Force Tips Blitzfire model # XX211A-HE oscillating portable lightweight high elevation attack monitor shall be supplied. The water powered turbine shall allow the nozzle to automatically sweep 10, 15 or 20 degrees left and right of center. The oscillating mechanism shall have a disconnect feature to allow for manual horizontal rotation.
The monitor shall include an integral safety shut off valve that will automatically shut down the water flow in the event control of the monitor is lost. The water flow is controlled by a six position detent, turbulence free slide valve. The detent flow control shall also function as the reset for the safety shut off valve to resume water flow.
The water inlet shall pivot up and down to allow for stability on uneven surfaces. The discharge has vertical elevation travel between 10 and 86 degrees above horizontal with a ball and socket joint. The same ball and socket joint shall allow for horizontal adjustment of 20 degrees left and right of center. 
The monitor shall have folding stabilization legs with carbide tips, which are concealed in the carry and stow positions. An integral carrying handle that also functions as an attachment point for the included anchor strap shall be provided. For corrosion resistance, the monitor shall be constructed from hardcoat anodized aluminum alloy and have a blue powder coat interior and exterior finish.
The monitor shall be configured with a 2-1/2" female NH swivel rocker lug coupling and 2-1/2" male NH rigid outlet with a flow rating of 500 GPM. The unit shall be covered by a five-year warranty.

