
1250 GPM MANUAL AERIAL PLATFORM MONITOR
Task Force Tips Monsoon model # Y4-M25A-P manually controlled aerial platform monitor shall be provided. The monitor shall operate with dual hand wheels which control stainless steel worm gears for rotation and elevation adjustment.
The monitor shall have the following capabilities:
Horizontal rotation with travel 90 degrees left and right of center;
90 degrees of vertical travel with stops at 45 degrees above horizontal and 45 degrees below horizontal;
Field changeable rotation stops at 45 and 90 degrees left and/or right of center and 30degrees above and/or 30 degrees below horizontal;
Flow capability of 1250 GPM with friction loss no more than 6 psi;
Maximum operating pressure of 200 PSI
For resistance to corrosion the monitor shall be constructed from hardcoat anodized aluminum with a silver powder coat interior and exterior finish. A threaded port for an optional pressure gauge shall be provided. The unit shall have a unique serial number and be covered by a five-year warranty.
The monitor shall be designed with a unique waterway that minimizes the path of travel, reduces friction loss and turbulence, and produces a far reaching water stream. The monitor shall be configured with a 4" ANSI 150 flange inlet and 2-1/2" male NH outlet.


