
1500 GPM FIXED LOCATION  MONITOR
Task Force Tips Typhoon, model # Y5-D21A-P aerial platform manually controlled monitor shall be provided. The Monitor shall operate with a hand wheel for rotation adjustment, and a hand wheel for elevation adjustment. 
The monitor shall have the following capabilities: 
Horizontal stops preset to 90 degrees left and right of center;
90 degrees of vertical travel from 45 degrees above horizontal to 45 degrees below horizontal;
Field changeable stops in 10 degree increments left and/or right and above and/or below horizontal;
Flow capability of 1500 GPM with no more than 19 PSI of  friction loss;
Maximum operating pressure of 200 PSI
For resistance to corrosion the monitor shall be constructed from hardcoat anodized aluminum with a silver powder coat interior and exterior finish. The discharge elbow shall include an integral stream straightener. A threaded port for an optional pressure gauge shall be provided. The unit shall have a unique serial number and be covered by a five-year warranty.
The monitor shall be configured with a 4" ANSI 150 flange inlet and 3-1/2" male NH outlet. 

