
WIRELESS REMOTE MONITOR CONTROL SYSTEM
Task Force Tips model # YE-RF-2400 wireless operator station for Monsoon, Hurricane, Typhoon, and Tornado series remote control monitors shall be installed.
The 2.4Ghz wireless transmitter shall be designed for remote usage and operate up to 500 feet away from the monitor-mounted receiver. The remote control includes membrane switches to control horizontal rotation, vertical elevation, nozzle stream pattern, stow, oscillate, auxiliary 1 and auxiliary 2. The handheld control enclosure shall be weatherproof, have a protective silicone rubber casing, a short, flexible antenna, four replaceable AA lithium batteries, a "Power" button with indicator LED, and low battery indication. The included radio receiver board is connected inside the monitor mounted control box. A storage bracket for the handheld control shall be included and shall comply with NFPA 1901 Section14.1.11. The unit shall be covered by a five-year warranty.
Location of storage bracket shall be:

