
FIXED LOCATION MONITOR
Task Force Tips Protector, model # Z1229A fixed location monitor shall be provided. The monitor shall be constructed from aluminum with stainless steel components and have a red powder coat internal and external finish. The monitor shall have a uniquely designed waterway that reduces the amount of turns the water makes and reduces friction loss for a more efficient flow.
The monitor shall be designed for simplicity of maintenance by requiring no grease and having seals, seats and swiveling elements that can be replaced in minutes with minimal tools. Two (2) 1/4" female NPT threaded ports shall be provided for installation of the optional drain and pressure gauge.
The monitor shall be configured with a 4" ANSI 150 companion flange inlet and 2-1/2" male Special thread (specify) discharge. The unit shall be covered by a five-year warranty.
Outlet thread shall be:

